# Project Sumup: Sorting robot with a twist

- Create an algorithm that allows a robot arm to identify and sort objects (e.g., colored cubes) into designated bins based on their color

- Use the Panda robot manipulator and assume to know the color of the objects

- For each detected object 𝑖:

  - Compute the picking position 𝐩pick,𝑖 and the placing position 𝐩place,𝑖 based on the object’s color

  - Use inverse kinematics to obtain the rob configurations 𝐪pick,𝑖 and 𝐪place,𝑖 .

- Control strategies to compare:

  - Joint-space trajectory interpolation: move from pick to place positions using interpolated joint trajectories between qpick,𝑖 and qplace,𝑖 .

  - Resolved-rate motion control: Use Cartesian velocity control to reach pick and place poses smoothly

- Analysis: plot and compare trajectories for both strategies (joint and Cartesian space), analyze end-effector path smoothness, execution time, and sorting accuracy.

Extra steps: mock a camera so that the robot has to find the location and colour of the spheres based on visual input. The sensing could be done in two ways:

- Usage of OpenCV to process the camera image, using HSV color masking to find the pixel coordinates of each berry.

- Using a Convolutional Neural Network that takes the raw camera image as direct input, and is trained to learn its own policy for “seeing” the berries.